A PROJECT REPORT ON

ALCOHOL DETECTON WITH VEHICLE  CONTROLLING

SUBMITTED TO THE UNIVERSITY OF PUNE, PUNE

IN THE PARTIAL FULFILLMENT FOR THE SUBJECT “ MINI PROJECT AND SEMINAR”

OF

TE – E&TC ( SEM – II )

BY

| SNEHA BHAMBURE | Exam. No. : | T80363036 |
| --- | --- | --- |
| SONALI BORKAR | Exam. No. : | T80363050 |
| KISHOR MITTAL | Exam No. : | T80363221 |

UNDER THE GUIDANCE OF

Prof. V.T. THITE

DEPARTMENT OF ELECTRONICS AND TELECOMMUNICATION ENGG.

STES’S

SMT. KASHIBAI NAVALE COLLEGE OF ENGINEERING

VADGAON BK., OFF. SINHGAD ROAD

PUNE 411041

2013 - 2014

B) CERTIFICATE

CERTIFICATE

This is to certify that the project report entitled

ALCOHOL DETECTION WITH VEHICLE CONTROLLING

Submitted by

| SNEHA BHAMBURE | Exam. No. : | T80363036 |
| --- | --- | --- |
| SONALI BORKAR | Exam. No. : | T80363050 |
| KISHOR MITTAL | Exam No. : | T80363221 |

is a bonafide work carried out by them under the supervision of Prof.  V.T.TITHE   and it is approved for the partial fulfillment of the requirement of University of Pune for the subject “ Mini Project and Seminar” of TE ( E&TC)

This project  report has not been earlier submitted to any other Institute or University for the award of any degree or diploma.

| Prof. V.T. TITHE | Prof. S.M. Ingawale | Prof. Dr.A.V.Deshpande |
| --- | --- | --- |
| Guide | Head | Principal |
| Department of E&T/C | Department of E&T/C | S.K.N.C.O.E,Pune-41 |
| S.K.N.C.O.E,Pune-41 | S.K.N.C.O.E,Pune-41 |  |

Place: Pune

Date :

ACKNOWLEDGEMENT

We would like to extend our acknowledgement to certain people who have been helpful and without whom this project would not have been a success .

We would express our profound gratitude to our project guide Prof. V.T. THITE in particular for her inspiring guidance which led to the development of the project.

We would  also like to thank Prof. S.M. Ingawale ,head of department ,E & tc for helping us in a successful completion of our project.

BY:

SNEHA BHAMBURE

SONALI BORKAR

KISHOR MITTAL

CONTENTS

CERTIFICATE

ACKNOWLEDGEMENT

LIST OF FIGURES 					I

LIST OF TABLES 					II

NOMENCLATURE 					III

| CHAPTER |  |  | TITLE | PAGE NO. |
| --- | --- | --- | --- | --- |
|  |  |  | ORGANIZATION OF REPORT |  |
| 1 |  |  | INTRODUCTION |  |
|  | 1.1 |  | INTRODUCTION OF PROJECT |  |
|  | 1.2 |  | OBJECTIVE OF PROJECT |  |
| 2 |  |  | LITERATURE SURVEY |  |
|  | 2.1 |  | INTRODUCTION |  |
|  |  | 2.1.1 | ISSUES |  |  | 1.3.3 | DESIGN ASPECT |  |
|  |  | 2.1.2 | APPROACH |  |
|  |  | 2.1.3 | OVERVIEW OF REPORT |  |
| 3 |  |  | DESIGN ASPECT |  |
|  | 3.1 |  | INTRODUCTION |  |

|  | 3.1 |  | INTRODUCTION |  |
| --- | --- | --- | --- | --- |
|  |  | 3.1.1. | BLOCK DIAGRAM DESCRIPTION |  |
|  |  | 3.1.2 | POWER SUPPLY DESIGN |  |
|  |  | 3.1.3 | POWER SUPLY DESIGN OF THE PROJECT |  |
|  |  | 3.1.4 | The reason to choose rectifier |  |
|  |  | 3.1.5 | Specification |  |
|  | 3.2 |  | FLOWCHART |  |
|  | 3.3 |  | CODE |  |
|  | 3.4 |  | SOFTWARE DESIGN |  |
| 4 |  |  | IMPLEMENTATION AND TESTING |  |
|  | 4.1 |  | CIRCUIT DIAGRAM |  |
|  | 4.2 |  | IMPLEMENTATION |  |
|  | 4.3 |  | PCB MAKING PROCESS SNAPSHOT |  |
|  | 4.4 |  | PCB LAYOUT |  |
|  | 4.5 |  | FINAL PROJECT SNAPSHOT |  |
| 5 |  |  | ADTVANTAGES , DISADVANTAGESAND APPLICATIONS |  |
|  | 5.1 |  | ADVANTAGES |  |
|  | 5.2 |  | DISADVANTAGES |  |
|  | 5.3 |  | APPLICATIONS |  |
| 6 |  |  | RESULT,DISCUSSION AND FUTURE SCOPE |  |
|  | 6.1 |  | RESULT |  |
|  | 6.2 |  | FUTURE SCOPE |  |
|  | 6.3 |  | DISCUSSION |  |
| 7 |  |  | REFERENCES |  |
|  |  | 7.1 | REFERENCE BOOK’S |  |
|  |  | 7.2 | WEB REFERENCES |  |
|  |  |  | BILL OF MATERIAL |  |
|  |  |  | DATASHEETS |  |

ORGANIZATION OF REPORT

This project consists of 7 chapters which gives us a brief idea about the project such as basic information design and implementation.

- INTRODUCTION

- LITERATURE SURVEY

- DESIGN ASPECT

- IMPLEMENTATION AND TESTING

- RESULT AND DISCUSSIONS

- ADVANTAGES,DISADVANTAGES AND APPLICATION.

- REFERENCE

1.     INTRODUCTION

In this chapter, overall study of the project is discussed .The motivation and objectives of the project has been discussed. General specifications of project are discussed.

2.        LITERATURE SURVEY

In this chapter the general introduction to the field of alcohol detection with vehicle controlling system with an overview of history of the same is discussed. The block diagram of each section is described briefly.

3.         DESIGN ASPECT

In this chapter all the design aspects are taken under consideration. The design and working of each block has been discussed in this chapter. The specifications and description of all the blocks are given in short. The software aspect of the system is discussed shortly in this chapter.

4         IMPLEMENTATION AND TESTING

In this chapter fabrication of PCBs and testing and troubleshoot condition of the system have been studied. The flowchart of PCB fabrication has been studied in this chapter.

5. RESULT AND DISCUSSION:

In this chapter, the final result after the successful working of the project has been  described with discussion of topics related to the project.

6. ADVANTAGES , DISADVANTAGES AND APPLICATIONS:

In this chapter, the merits and demerits of the project has been discussed considering various application and future scope of the project.

7.REFERENCES:

It includes a list of  all books, software used in designing, implementation of our project . All website referred are also listed below.

CHAPTER 1

INTRODUCTION

  - INTRODUCTION OF THE PROJECT

It gives us immense happiness to present  before you the project which is ‘ALCOHOL DETECTION WITH VEHICLE CONTROLLING’ as part our curriculum for T.E in

E & TC. This   project  report  is  used  for  detecting  the  alcohol  drunken  people Software requirement is Protieus. This project is also helpful in detecting the exact consumption of alcohol taken by the person. This will  help  in taking strict action required at that moment.

So  for  detecting alcohol  drunken  people  , we  have  to  use  microcontroller(89S52),  alcohol  sensor(MQ3),  buzzer,  relay , ADC, LCD.

As part of the SAFETEA-LU legislation, the Secretary of Transportation was directed to conduct a study on the potential for reducing the incidence of alcohol-related motor vehicle crashes and fatalities through advanced vehicle-based alcohol detection systems.

Also included is an assessment of the practicability and effectiveness of such systems.[6] NHTSA’s Office of Human-Vehicle Performance Research has tasked the Volpe National Transportation Systems Center (Volpe Center) to assess the potential for vehicle-based technologies to prevent alcoholimpaired crashes.

The purpose of this TOPIC report is to identify vehicle-based technologies capable of detecting and preventing alcohol impaired driving. Research data will provide input  to a report required to be submitted in 2007 to Congress.

1.2 OBJECTIVES OF  THE  PROJECT

The main objective of alcohol detector circuit project is to detect the alcohol drunken people. The basic idea  behind the project is to reduce the drink and driving cases in city with alcohol detectors incorporated  in vehicles. Now a days many accidents are reported. The percentage has been increased to a  great level.

Another objective of this project is to provide a economical solution in the field of alcohol solution . Therefore our project is of great help in preventing drink and drive accidental case.

The National Highway Traffic Safety Administration's Office of Human-Vehicle Performance

Research tasked the Volpe National Transportation Systems Center (Volpe Center) of the U.S.

Department of Transportation’s Research and Innovative Technology Administration to identify

current and emerging vehicle-based technologies that can detect driver blood alcohol concentration (BAC) and monitor driver impairment due to alcohol. Detection technologies have the potential to prevent death and injury by monitoring BAC and/or driving performance for signs of impairment, and if necessary either prevent ignition or take other actions to prevent a crash.

As part of the SAFETEA-LU legislation, the Secretary of Transportation was directed to conduct a study on reducing the incidence of alcohol-related motor vehicle crashes and fatalities through research on advanced vehicle-based alcohol detection systems, including

an assessment of the practicability and effectiveness of such systems. In support of this

mandate, this report on Technology to Prevent Alcohol Impaired Crashes (TOPIC) assesses

the capability of existing and anticipated technologies to detect and prevent alcohol-impaired

driving. It also includes a concept of operations to describe how to implement technology-based

countermeasures while addressing concerns such as privacy, public acceptance, and legal issues.

CHAPTER 2

LITERATUTRE SURVEY

2.1 INTRODUCTION

2.1.1 ISSUES

- In 2004, the National Highway Traffic Safety Administration reported that there were 16,694deaths and 248,000 people injured as a result of alcohol-related motor vehicle crashes.[2, 3]

- Alcohol-related motor vehicle fatalities account for 39 percent of all motor-vehicle-related deaths.

- The NHTSA Administrator has stated that this fatality rate is a national concern during

her 2006 testimony to Congress, referring to the provisions in the Safe, Accountable,                         Flexible, Efficient Transportation Equity Act: A Legacy For Users (SAFETEA-LU) legislation to provide  increased funding to reduce impaired driving.

- Fatalities and injuries due to motor vehicle crashes create particularly heavy losses to society when expressed as disability-adjusted life-years (DALYs), because the motor vehicle crash victims tend to be young. (DALYs tabulate the number of years lost to premature death and disabling injuries.) The public health community is increasingly aware of the losses world-wide due to vehicle accidents and forecasts that vehicle accidents will move from the ninth cause of DALYS in 1990 to the third leading cause by 2020. [4] The 16 to 20 and 20 to 24 age groups have the highest fatality rate per 100,000 -- more than double the rate for the overall population,[5] with a substantial proportion of these crashes being alcohol-related.

    - APPROACH

- Impairment detection technologies have the potential to prevent serious crashes by stoppingimpaired drivers from starting or operating vehicles.

- Technology can detect driver BAC and lock out the driver or monitor driving performance for signs of impairment.

- If driver impairment is evident, the system can warn the driver and or impose any of a range of measures to mitigate risk..

- This report assesses the ability of technologies, existing and anticipated, to detect driver

- impairment from alcohol and identifies international state-of-the-art vehicle-based technology options to prevent alcohol-impaired automotive crashes.

- This analysis was carried out with the support of the Intelligent Vehicle Initiative (IVI) program, created in 1998 as part of the Department of Transportation ITS program.

- The IVI program focuses on the collision warning system as an effective tool to reduce the number of accidents by providing effective and timely  warnings to drivers.

- The Volpe Center team interviewed stakeholders and interested parties and reviewed research  results to assess the capability of vehicular technologies to reduce alcohol-impaired driving.

- The team acquired academic expertise both to review post-1995 literature and to identify international sources of expertise for vehicle-based alcohol impairment detection.

- The research team collaborated with European experts to acquire first-hand information about the results of the EU projects: System for the Effective Assessment of the Driver State and Vehicle Control in Emergency Situations (SAVE) and System for Effective Assessment of Driver Vigilance and Warning According to Traffic Risk Estimation (AWAKE), the plans and intent of the EU project Advanced Sensor Development for Attention, Stress, Vigilance and Sleep/Wakefulness Monitoring (SENSATION), and the status of relevant research by European institutes on vehicle-based alcohol impairment identification and countermeasures.

- In addition, the European collaborator conducted personal interviews with six major European stakeholders representing technology developers such as original equipment manufacturers and suppliers and EU government agencies about vehicle-based alcohol impairment identification and countermeasures.

2.1.3  OVERVIEW OF REPORT

- This report describes the most effective means of measuring alcohol impairment and the strategies for implementing technology-based countermeasures.

- The report estimates the potential for crash reductions as a result of introducing TOPIC in relation to fatalities and injuries avoided; describes the strengths and weaknesses of near-term approaches, including breath alcohol ignition interlock devices (BAIID) for DUI offenders; evaluates current and emerging crash avoidance technologies; identifies cross-cutting implementation issues likely to accompany the introduction of these technologies; identifies research needs; and provides a concept of operations using promising technologies.

CHAPTER 3

DESIGN ASPECT

3.1 INTRODUCTION

Here we have discussed the basic block diagram of the alcohol detector as shown below.

3.1.1  BLOCK  DIAGRAM  DESCRIPTION

a)LIQUID CRYSTAL DISPLAY:

- LCD is used in a project to visualize the output of the application.

- We have used 16x2 LCD which indicates 16 columns and 2 rows. So, we can write 16 characters in each line. So, total 32 characters we can display on 16x2 LCD.

- LCD can also used in a project to check the output of different modules interfaced with the microcontroller.

- Thus LCD plays a vital role in a project to see the output and to debug the system module wise in case of system failure in order to rectify the problem.

b) BUZZER:

- Buzzer or beeper is an audio signaling device, which may be mechanical, electromechanical, or piezoelectric.

- Typical uses of buzzers and beepers include alarm devices, timers and confirmation of user input such as a mouse click or keystroke.

c) ALCOHOL SENSOR:

- Alcohol sensor is used to detect weather the person have consumed the alcohol or not.

- The sensor which we have used is MQ3 sensor.

- HOW MQ3 SENSOR WORK’s:

- MQ3 sensor is an analog sensor and the output is in analog form.

- So the output of this sensor is provided to the input of the ADC which will convert this analog output in digital input to the microcontroller.Sensitive material of MQ-3 gas sensor is SnO2, which with lower conductivity in clean air.

- When the target alcohol gas exist, The sensor’s conductivity is more higher along with the gas concentration rising.

- Please use simple electro circuit, Convert change of conductivity to correspond output signal of gas concentration.MQ-3 gas sensor has high sensitivity to Alcohol, and has good resistance to disturb of gasoline, smoke and vapour.

- The sensor could be used to detect alcohol with different concentration; it is with low cost and suitable for different application

3.1.2 POWER SUPPLY:

- The basic step in the designing of any system is to design the power supply required for that system.

- The steps involved in the designing of the power supply are as follows,

) Determine the total current that the system sinks from the supply.

2) Determine the voltage rating required for the different components.

- The bridge rectifier and capacitor i/p filter produce an unregulated DC voltage which is applied at the I/P of 7805.As the minimum dropout voltage is 2v for IC 7805, the voltage applied at the input terminal should be at least 7 volts .

- C1 (1000 µf / 65v)is the filter capacitor and C2 and C3 (0.1 pf) is to be connected across the regulator to improve the transient response of the regulator.

- Assuming the drop out voltage to be 2 volts, the minimum DV voltage across the capacitor C1 should be equal to 7volts (atleast).

3.1.3  Power supply design of the Project :

- The average voltage at the output of a bridge rectifier capacitor filter combination is given by

- Vin(DC) = Vm – Idc / 4 f C1

- Where , Vm=√2 Vs  and Vs = rms secondary voltage

- Assuming Idc to be equal to max. load current, say 100Ma

C = 1000 uf / 65v , f=50hHz

15 = Vm – 0.1 / 4*50*1000*10¯6

15= Vm – 0.1 /  0.2

Vm=15.5 volts

- Hence the RMS secondary Voltage

- Vrms = Vm / √2

= 15.5 / √2

=15.5 / 1.41421

=10.9601 volts

- So we can select a 15v secondary Voltage In our system most of the components require 5 V as operating voltage such as micro controller, MAX 232, LCD etc.

- The total current, which our circuit sinks from the power supply, is not more than 100 mA. We have used Regulator IC 7805 that gives output voltage of 5V.

- The minimum input voltage required for the 7805 is near about 7 v. Therefore we have used the transformer with the voltage rating 230v-10v and current rating 500/750 mA.

- The output of the transformer is 12 V. This voltage is converted into 12 V DC by Bridge rectifier circuit.

3.1.4 The reasons for choosing the bridge rectifier are

- The TUF is increased to 0.812 as compared the full wave rectifier.

- The PIV across each diode is the peak voltage across the load =Vm, not 2Vm as in the two diode rectifier

- Output of the bridge rectifier is not pure DC and contains some AC some AC ripples in it.

- To remove these ripples we have used capacitive filter, which smoothens the rippled out put that we apply to 7805 regulators IC that gives 5V DC.

- We preferred to choose capacitor filters since it is cost effective, readily available and not too bulky.

    - Microcontroller (89S52) :

SPECIFICATIONS

- HARDWARE:

Features

• Compatible with MCS-51® Products

• 8K Bytes of In-System Programmable (ISP) Flash Memory

– Endurance: 1000 Write/Erase Cycles

• 4.0V to 5.5V Operating Range

• Fully Static Operation: 0 Hz to 33 MHz

• Three-level Program Memory Lock

• 256 x 8-bit Internal RAM

• 32 Programmable I/O Lines

• Three 16-bit Timer/Counters

• Eight Interrupt Sources

• Full Duplex UART Serial Channel

• Low-power Idle and Power-down Modes

• Interrupt Recovery from Power-down Mode

• Watchdog Timer

• Dual Data Pointer

• Power-off Flag

• Fast Programming Time

• Flexible ISP Programming (Byte and Page Mode)

- SOFTWARE:

- Keil uV2 Software for Programming

PIN DIAGRAM FOR MICROCONTROLLER:

3.2 FLOWCHART

START

INITIALIZE  LCD

DISPLAY PROJECT NAME

SELECT CHANNEL 1 11

READ  ADC

STORE AND DISPLAY THE ALCOHOL ON LCD

IS ALCOHOL

GREATER THAN 90

RELAY ON

BUZZER OFF

RELAY OFF

BUZZER ON

Yes                                                                                      NO

Fig no 2: Microcontroller circuit diagram

3.3 CODE:

LCDRS   BIT     P1.0

LCDRW   BIT     P1.1

LCDEN   BIT     P1.2

LCDDATA EQU     P0

R_D     BIT     P3.7    ;2

W_R     BIT     P3.6    ;3

INTR    BIT     P3.5    ;5

ADCPORT EQU     P2

BUZZER  bit     P1.3

RELAY	bit	P3.4

CM      BIT     0

bit1    bit     1

CNT     EQU     30H

delr0    equ     40h

delr1    equ     41h

delr2    equ     42h

delr3    equ     43h

CMP    equ     44h

org 0000h

ajmp    over

org    0050h

over:

//MSG1:   DB      'Alcohol Detection'

//MSG2:   DB      '  in car         '

SETB	RELAY

setb    buzzer

ACALL   LCDINIT

ACALL   CLEAR

mov     dptr,#MSG1

mov     R7,#16

ACALL   DISP_MSG

MOV     A,#0C0H

ACALL   COMMAND

mov     dptr,#MSG2

mov     R7,#16

ACALL   DISP_MSG

ACALL   DELAY

AGN:

agn1:

ACALL   READ_ADC

ACALL   DISPPARA

MOV     A,CMP

mov	b,#2

div	ab

cjne    a,#30,x1

x1:     jc      nomsg

mov     a,#0c0h

acall   command

mov     dptr,#MSGg	//msgg:  db      'Alco. Detected    '

mov     R7,#16

ACALL   DISP_MSG

clr     buzzer

acall   delay

acall   delay

acall   delay

acall   delay

CLR	RELAY

x2:

ACALL   DELAY

AJMP    AGN

nomsg:

clr     bit1

acall   delay

SETB	RELAY

setb    buzzer

AJMP    AGN

msgg:  db      'Alco. Detected    '

READ_ADC:

MOV     ADCPORT,#0FFH	//P2

CLR     W_R			//SOC

NOP

NOP

NOP

NOP

NOP

NOP

SETB    W_R

SETB    INTR	//EOC

HRE:    JB      INTR,HRE

CLR     R_D

NOP

NOP

NOP

NOP

MOV     A,ADCPORT

SETB    R_D

MOV     CMP,A

RET

DISPPARA:

ACALL   CLEAR

mov     dptr,#LEVL

mov     R7,#16

ACALL   DISP_MSG

MOV     A,#89H

ACALL   COMMAND

MOV     A,CMP

mov	b,#2

div	ab

ACALL   HTB

mov	a,#' '

ACALL   DISPLAY

mov	a,#'m'

ACALL   DISPLAY

mov	a,#'l'

acall  DISPLAY

RET

HTB:

MOV     B,#64H

DIV     AB

ACALL   DISPLAY1

MOV     A,B

MOV     B,#0AH

DIV     AB

ACALL   DISPLAY1

MOV     A,B

ACALL   DISPLAY1

RET

LEVL:    DB     'Alcohol:        '

lcdINIT:

mov a,#38h

acall command

mov a,#0eh

acall command

mov a,#06h

acall command

clear:

mov a,#01h

acall command

ret

;lcd strobe subroutine

command:

acall ready

mov LCDDATA,a

clr LCDRS

clr LCDRW

setb LCDEN

clr LCDEN

ret

display1:

add    a,#30h

display:

acall ready

mov LCDDATA,a

setb LCDRS

clr LCDRW

setb LCDEN

clr LCDEN

ret

ready:

clr LCDEN

mov LCDDATA,#0ffh

clr LCDRS

setb LCDRW

;----------------

wait:

clr LCDEN

setb LCDEN

jb LCDDATA.7,wait

clr LCDEN

ret

disp_msg:

mov     r6,#00h

LO1:     MOV     A,R6

MOVC    A,@A+DPTR

ACALL   DISPLAY

INC     R6

DJNZ    R7,LO1

RET

delay:  mov DELR0,#10

AJMP    HER2

DB_delay:mov DELR0,#4

her2:  mov DELR1,#254

her1:  mov DELR2,#254

her:   djnz DELR2,her

djnz DELR1,her1

djnz DELR0,her2

ret

MSG1:   DB      'Alcohol Detection'

MSG2:   DB      '  in car         '

end

3.4    SOFTWARE DESIGN

MIDE tool for the 8052 Microcontroller Architecture support every level of software developer from the professional applications engineer to the student just learning

about software development.

CHAPTER 4

IMPLEMENTATION AND TESTIN

4.1   CIRCUIT DIAGRAM:

4.2    IMPLEMENTATION

- The most important requirement of this project was to build a PCB with minimum weight and size. A zero PCB is a drilled board. Drilling process removes a lot of material from board and weight is reduced .Designing procedure is as follows:

i.        Decide a proper place for components

ii.       Actual placements of components from zero PCB

iii.      Connecting tracks with solid wires

4.3    PCB MAKING PROCESS SNAPSHOTS:

4.4   PCB LAYOUTS

4.5 FINAL PROJECT SNAPSHOTS

PCB LAYOUT :

- Layout basically means placing or arranging things in a specific order on the   PCB.

- Layout means placing of components in an order. This placement is made such that the interconnection lengths are optimal .

- At the same time, it also aims at providing accessibility to the components for insertion testing and repair.

- The PCB layout is the starting point for the final artwork preparation layout design should reflect the concept of final equipment.

- There are several factors, which we must keep in mind for placing the layout.

Schematic Diagram:

- The schematic diagram forms main input document for preparation of the layout  For this purpose the software for PCB design, ORCAD was used.

Electrical and thermal requirement:

- The PCB designer must be aware of the circuit performance in critical aspects of the same concerning electrical conditions and the environment to be used in.

Mechanical requirement:

- The designer should have the information about physical size of the board, type of installation of board (vertical/horizontal). The method of cooling adopted, front panel operated components etc.

Component placing requirement:

- All components are to placed first in a configuration that demands only the minimum length for critical conductors. These key components are placed first and the others are grouped around like satellites.

Components mounting requirements:

- All components must be placed parallel to one another as far as possible .i.e. in the same direction and orientation mechanical over stressing of solder should be avoided.

Layout Methodology:

- For proper layout design minimal steps to be followed are:

    - Get the final circuit diagram and component list.

    - Choose the board types, single sided / double sided / multilayer

    - Identify the appropriate scale for layout.

    - Select suitable grid pattern.

    - Choose the correct board size keeping in view the constraints.

    - Select appropriate layout technique, manual / automated.

    - Document  in the form of the layout scale.

CHAPTER 5

ADVANTAGES, DISADVANTAGES AND APPLICATION

5.1   ADVANTAGES

- Less time delays

- Quick response time

- Fully automate system

- Robust system

- Low power requirement

- Efficient

- Economical

- Time saving kit

- Portable

- It requires less space

- It can provide an automatic safety system for caes and other vehicles.

5.2    DISADVANTAGES

- Limited range.

- If any person other than the driver has consumed alcohol and if the system detects it even then it will not allow the car to start.

- Due to heating of coil in the sensor , the sensor takes time to initialize.

5.3 APPLICATIONS

    - Drunk driving detection system is incorporated in vehicles.

    - Alcohol detector project  can be used in various vehicles for detecting whether the driver has consumed alcohol or not

    - This project can also be used in various companies or organization to detect alcohol consumption of employees.

    - It can implement GSM technology to inform the relatives or owners of the vehicle about the alcohol consumption.

    - We can implement GPS technology to find the location of the

vehicle

CHAPTER6

RESULT , DISCUSSION AND FUTURE SCOPE

6.1  RESULT:

The main components in the project are described along with working  which is useful to understand the project better and helps us in analyzing the scope and working.

Thus , the set point for alcohol consumption has been adjusted to 90ml.

As soon as the alcohol level in the vehicle crosses the set point , the buzzer starts buzzing which is an indication of danger . Similarly , as soon as the alcohol level in the vehicle drops below the set point , the buzzer automatically switches off.

6.2 FUTURE SCOPE:

- In accident cases  it is a very useful.

- It is useful in identifying the culprit.

- By modifying the circuit we can use it at professional level .

- GSM technology can be implemented

6.3   DISCUSSION:

Blood alcohol concentration (BAC) is considered to be standard for measuring the degree  to which an individual is impaired by alcohol .For years the study have shown that there is a direct correlation between the blood alcohol concentration  and the degreevto which reactions and judgements are impaired.the methodology are used for blood alcohol testing is called gas chromatography.

6.2 CONCLUSION:

- We have successfully implemented ALCOHOL DETECTION WITH VEHICLE CONTROLLING

- With the help of this  project the human efforts are largely reduced.

- It is the advanced technology which is rising rapidly

CHAPTER7

REFERENCES

7.1 REFERANCES    BOOKS

Muhammad Ali   Mazidi and Janice  Gillispi  Mazidi, ”The 8051 Microcontroller and Embedded systems”.

7.2       WEBREFERENCES

- http://www.datasheetcatalog.org/SGSThomsonMicroelec/mXtyyvx.pdf

- http://www.8051projects.info/exp9.asp

- http://www.swedetrack.com/obstact.htm

BILL OF MATERIAL:

| Name | Specification | Quantity | Prize |
| --- | --- | --- | --- |
| Liquid crystal display | 16*2 | 1 | 150 |
| Ceramic Capacitor | 33pf | 4 | 4 |
| Diode |  | 5 | 25 |
| Oscillator | 11.0592Mhz | 1 | 11 |
| Transistor | BC547 | 1 | 4 |
| Electrolytic capacitior |  | 2 | 6 |
| Alcohol sensor |  | 1 | 400 |
| 8051(AT89s51) ADC | 0804 | 1 1 | 75 50 |
| Transformer |  | 1 | 200 |
| Resistor | 1k | 1 | 2 |
| Resistor: | 100k | 1 | 3 |
|  | 2.3k | 2 | 6 |
|  | 10k | 2 | 8 |